## 🔌 Hub vs Switch (Complete Explanation – Easy + Technical)

Both Hub and Switch are networking devices used in LAN (Local Area Network), but they work very differently.

## 🔹 1️⃣ Hub

A Hub is a simple networking device.

### 📌 How it works:

- When one computer sends data,

- The hub broadcasts (sends) that data to all connected devices.

- Every device receives it, but only the target device accepts it.

### 📊 Key Features:

- Works on Physical Layer (Layer 1) of OSI model

- No intelligence (cannot check MAC address)

- Same bandwidth shared by all devices

- More network collisions

- Slower performance

### 📌 Example:

If PC1 sends data to PC3, hub sends it to:
➡ PC2
➡ PC3
➡ PC4
Everyone receives it.

## 🔹 2️⃣ Switch

A Switch is a smarter networking device.

### 📌 How it works:

- It checks the MAC address

- Sends data only to the target device

- Other devices do not receive that data

### 📊 Key Features:

- Works on Data Link Layer (Layer 2) of OSI model

- Maintains MAC Address Table

- Less collisions

- Better speed

- Dedicated bandwidth per device

### 📌 Example:

If PC1 sends data to PC3, switch sends it only to:
➡ PC3
Not to PC2 or PC4.

## 🔥 Quick Comparison Table

| Feature | Hub | Switch |
| --- | --- | --- |
| OSI Layer | Layer 1 | Layer 2 |
| Data Sending | Broadcast to all | Sends to specific device |
| Speed | Slow | Fast |
| Collision | More | Less |
| Intelligence | No | Yes |
| Bandwidth | Shared | Dedicated |

## 🎯 Simple Definition

- Hub = Dumb device (broadcasts to everyone)

- Switch = Smart device (sends to exact device)